Simulation of Solar System

Author: Ian Chang, Yi-Jhan Huang

Abstract This is a simulation and analysis project based on python and pygame for the purpose of a visual representation of the solar system.

1. Introduction

Our research project is based on exploring the solar system in the way of using python and other few packages (numpy, pygame, matplotlib) to simulate and analyze the orbital period, path, and other related details about how each planet react with the sun if different attributes have changed.

2. Data and Methods

Programming Language: Python

Packages required: Numpy, Pygame, Matplotlib

We create our self-defined class “Planet” as our main class for simulation. With the combination of visual representation using pygame and graphic analysis using matplotlib, we get all of our result based on different physics equation and the actual mass, distance to sun for each planet to complete all the simulation.

Class Planet (Complete code in appendix 1)

- Constructor:

def__init__('name', distance to sun(astronomical unit),

radius (self-define for graphic simulation),

presenting color (RGB code),

mass of planet (Kilogram)):

- Attraction Function: (use as the function to calculate attraction force between two [class planet] objects)

def attraction(self, other):

return force_x, force_y

Physics Equation relates to the function

First equation (left): two-dimension distance formula

Second equation (right): Newton’s law of universal gravitation

- Update Position Function: (use as the function to update position of the planet. Calculated from a list of Planet objects)

def update_pos(self, planets):

return None

Physics Equation relates to the function

First equation (left): Newton's Second Law of Motion

Second equation (right): instantaneous acceleration formula

- Move function: (use as the function to re-scale the point within the orbit position list to fit in the screen of the simulation, returning a list of orbit points and final x, y position)

def move(self):

return updated_points, (x, y)

Simulation (Complete code in appendix 2)

def simulation(planets, limit=-1, delay=True):

pygame.init()

...

while run and count != limit:

...

pygame.quit()

Parameters for the function:

planets: list of planets in the system

limit: set limit for move steps

delay: (Boolean) if True, there will be delay in the update of the graph

3. Results

- Normal Case (The actual Solar system, mass of sun = 1.98892e30)

Resulting Path, 3d representation

Analyze:

- No clear change of the orbital path

- We can see that the planet further from the sun has a longer orbit period

Resulting Path, 2d representation

Analyze:

- Not changing the average distance from the sun

- The distance change in wave, indicates the orbit path is ellipse instead of circle

Resulting Path, 2d representation

Analyze:

- Sun is actually slowly moving away from the origin due to attraction force from other planets in the solar system

- The distance change in wave, indicates the orbit path is ellipse instead of circle

- The wave length is smaller for planet closer to the sun, indicates that the orbit speed is faster

- What if Case (Mass of the sun / 100, mass of sun = 1.98892e28)

Analyze:

- Mercury, Venus, Earth is slowly leaving the origin

- Mars is slowly moving closer to the origin

- What if Case (Mass of the Sun = Earth, mass of sun = 5.972e24)

Analyze:

- Mercury, Venus, Earth, Mars are not orbiting around the sun.

- Gravitational force slowly pulls each planet together

- What if Case (Mass of the Sun = 1, mass of sun = 1)

Analyze:

- Mercury, Venus, Earth, Mars are not affected by sun.

- Gravitational force slowly pulls each planet together stronger

- What if Case (Mass of the Sun * 100, mass of sun = 1.98892e32)

Original Mass (left), *100 Mass (right)

Analyze:

- Earth is orbiting closer and at a faster speed.

4. Conclusion

We successfully simulate the solar system using the pygame and numpy. As for the analysis part, the result suggests that the orbit path of the planets is actually in ellipse instead of circle. Also, from all the what if case of the changing of mass of the sun, we can see that the orbiting of the planet is strongly based on the large mass of the sun and the attraction force creates from the extremely large mass of the sun.

5. References

- https://towardsdatascience.com/simulate-a-tiny-solar-system-with-python-fbbb68d8207b

- https://thepythoncodingbook.com/2021/12/11/simulating-3d-solar-system-python-matplotlib/

- https://towardsdatascience.com/simulate-a-tiny-solar-system-with-python-fbbb68d8207b

- https://github.com/xhinker/orbit/blob/main/solar_orbit_3d_plt.py

Appendix 1

class Planet:

AU = 149.6e6 * 1000             # distance from earth to sun

G = 6.67428e-11                 # gravitational constant

SCALE = 200 / AU                # small value to scale down the solar system to fit it on the screen

TIMESTEP = 60 * 60 * 24         # 1 day

MASS_OF_SUN = 1.98892 * 10 ** 30 # mass of sun

WIDTH, HEIGHT = 800, 800

# Constructor

def __init__(self, name, dis_to_sun, radius, color, mass):

self.name = name

self.x = -dis_to_sun * Planet.AU           # x position

self.y = 0                              # y position

self.radius = radius                    # radius

self.color = color                      # color

self.mass = mass                        # mass

self.sun = (dis_to_sun == 0)           # if it's the sun

self.dis_to_sun = dis_to_sun * Planet.AU  # distance from planet to sun

if (self.sun): Planet.MASS_OF_SUN = mass

self.orbit = []

self.x_speed = 0 # x speed

self.y_speed = 0 if (self.sun) else math.sqrt((Planet.G * Planet.MASS_OF_SUN) / self.dis_to_sun)     # y speed

# Draw line function for the updated_points

def move(self):

# divide by two because pygame (0, 0) is the top left corner

x = self.x * self.SCALE + self.WIDTH / 2     # x position on screen

y = self.y * self.SCALE + self.HEIGHT / 2    # y position on screen

updated_points = []

for point in self.orbit:

x, y = point

x = x * self.SCALE + self.WIDTH / 2

y = y * self.SCALE + self.HEIGHT / 2

updated_points.append((x, y))

return updated_points, (x, y)

def attraction(self, other):

distance_x = other.x - self.x # distance between x positions

distance_y = other.y - self.y # distance between y positions

distance = math.hypot(distance_x, distance_y) # distance between planets

force = self.G * self.mass * other.mass / distance ** 2 # force between the planets

theta = math.atan2(distance_y, distance_x) # angle between the planets x and y positions

force_x = math.cos(theta) * force # force in the x direction

force_y = math.sin(theta) * force # force in the y direction

return force_x, force_y

def update_pos(self, planets):

total_fx = total_fy = 0 # initialize x and y force to zero

for planet in planets:

if self == planet: # if the planet is itself

continue

fx, fy = self.attraction(planet)

total_fx += fx

total_fy += fy

self.x_speed += total_fx / self.mass * self.TIMESTEP

self.y_speed += total_fy / self.mass * self.TIMESTEP

self.x += self.x_speed * self.TIMESTEP

self.y += self.y_speed * self.TIMESTEP

self.orbit.append((self.x, self.y))

def __str__(self):

return f"""Planet(name={self.name}, radius={self.radius}, color={self.color}, mass={self.mass}, dis_to_sun={self.dis_to_sun},

x={self.x}, y={self.y}, sun={self.sun}, orbit_list={self.orbit[0:5]}, x_speed={self.x_speed}, y_speed={self.y_speed})

"""

Appendix 2

def simulation(planets, limit=-1, delay=True):

pygame.init()

# set pop up screen size

SCREEN = pygame.display.set_mode((Planet.WIDTH, Planet.HEIGHT))

FONT = pygame.font.SysFont("garamond", 16)

# set title of window

pygame.display.set_caption("Solar System")

run = True

clock = pygame.time.Clock()

count = 0

while run and count != limit:

SCREEN.fill((0, 0, 0)) # fill the screen black every frame, or else the old frames will still be on the screen

# Quit loop when receive quit signal

for event in pygame.event.get():

if event.type == pygame.QUIT:

run = False

for planet in planets:

planet.update_pos(planets) # update the position of the planets using the forces of attraction

updated_points, (x, y) = planet.move() # draw the planets

if len(updated_points) >= 2: pygame.draw.lines(SCREEN, planet.color, False, updated_points)

if not planet.sun:

planet_name_text = FONT.render(f"{planet.name}", True, WHITE)

SCREEN.blit(planet_name_text, \

(x - planet_name_text.get_width() / 2, y - planet_name_text.get_height() / 2 - 20))

pygame.draw.circle(SCREEN, planet.color, (x, y), planet.radius) # draw the planet

pygame.display.update() # update the display

if (delay): clock.tick(60) # Set up update delay

count += 1

pygame.quit()